PiccoLink driver control ( ActiveX )

The PiccoLink RF500 driver control is a 32-bit Windows ActiveX control  ( PLOCX2.ocx ), which controls the data communication between the basestation and the HOST application.

PLOCX2 makes HOST system software development easy because it does all the dirty work for you. The use of PLOCX2 provides a lot of functionality and code that you don't have to write or debug. All you have to do is to figure out the entry points and how to use them.

You can use PLOCX2 control in applications that are developed with Visual Basic , Visual C++, Excel, Access , FoxPro and many more products who support ActiveX controls.

Rx

Tx

PiccoLink®

#### RS232

Methods & Events

### PLOCX2

USER

APPLICATION

External Basestation Data Source

PLOCX2 can be connected with the basestation using the serial port. PLOCX2 can also receive basestation data from another data source (eg. from a network).

## Features

- Communication traffic and transaction details of every Hand terminal are visually seen.

- Serves Hand terminal data via events.

- Automatic CRC checking.

- Automatic resend handle.

- Easy frame building methods.

Methods:	Ack( )	Bell( )	Button( )	ClearCmd( )	ClearSendBuffer( )

CmdSet(  )	Connect( )	DataIn( )	FieldCmd( )	FldTxt( )

GetData( )	GetExtraID( )	IsData( )	NewField( )	PopMessage( )

Send( )	SetView( )	Text( )	What( )	GetRSSI( )

GetLastFrameID( )	ClearForm( )

Events:	DataArrived( )		Properties:	BoxPerRow

DataFromHsField( )		Use <DataFromHsField>

DataOut( )

What( )

Methods and events for data transfer between the driver control and the user application:

Connect( short comport_number ); ( METHOD )

Connect the driver control to the specific serial port. Connect( 0 ) disconnect.

DataIn( short FAR* data, short data_len, short source_addr); ( METHOD )

The data of the basestation can be delivered to the driver control by this method.

data = data from the basestation.

data_len = data length

source_addr = User defined data source.

DataOut( short FAR* data,short data_len, short dest_addr);  ( EVENT)

When the data from the basestation have been delivered to the driver control using the DataIn method, the data to the basestation will be offered by this event.

data = data to the basestation

data_len = data length

dest_addr = Destination address. Defined in DataIn method (source_addr).

DataFromHsField( long id, short frameid,short pos, LPCTSTR data, short data_len); ( EVENT, can be set ON/OFF)

This event offers a string from a specific Hand Terminal, from a specific form and from a specific position.

id = Hand Terminal ID (1-65535)

frameid = User defined number of form. If the frameid is -1, the data are from the initial screen or from a function key.

pos = position of inputfield. ( 0-239 )

data = data from inputfield

data_len = data length

DataArrived( long id, short frameid); ( EVENT )

This event occurs after the DataFromHsField events

id = Hand Terminal ID (1-65535)

frameid = The number of form defined by the user. If the frameid is -1, the data comes from the initial screen or from a function key.

BSTR GetData( long id, short pos); ( METHOD )

This method will return a string from the inputfield pos.

id = Hand Terminal ID (1-65535)

pos = Position of inputfield. ( 0-239 )

BOOL IsData( long id, short pos); ( METHOD )

This method will return TRUE if data is coming from pos.

id = Hand Terminal ID (1-65535)

pos = Position of inputfield. ( 0-239 )

BOOL GetExtraID( long id ) ( METHOD )

This method returns Extra ID of the Hand Terminal.

id = Hand Terminal ID (1-65535)

BOOL Send(long id, short frameid); ( METHOD )

This method will send the form to the Hand Terminal via com port or DataOut event.

id = Hand Terminal ID (1-65535)

frameid = The number of form defined by the user. If frameid is -1 , the previously defined frameid is valid.

short GetRSSI( long id ); ( METHOD )

This method returns RSSI level of the last received message.

id = Hand Terminal ID (1-65535)

short GetLastFrameID( long id ); ( METHOD )

This method returns last sended frameid [ Send() method parameter 2 ].

id = Hand Terminal ID (1-65535)

# Methods of Form creation:

These methods fill the driver control send buffer. When the form is built with these methods, Send( ) method will send the form to the Hand Terminal via com port or DataOut event. Please see Communication Protocol Manual for details.

BOOL ClearForm( long id ); ( METHOD )

Clears text and field from display and sets view to row 0.

Id= hand Terminal ID (1-65535)

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL Ack( long id ); ( METHOD )

This method adds the Ack command to the send buffer. Ack is used at NewField command. ( please see newfield style bits ).

id = Hand Terminal ID (1-65535)

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL Bell( long id ); ( METHOD )

Generates a beep

id = Hand Terminal ID (1-65535)

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL Button( long id, short pos, LPCTSTR txt); ( METHOD )

Creates a "button" style field . The BUTTON method will create a locked field filled with text. Field text will be sent to the HOST by pressing the OK key.

This method is suitable for menu creation.

SND_ALL and NO_SEND functions are disabled. (please see NewField() method )

To create a button field with SND_ALL and NO_SEND functions, use the NewField() and FldTxt() methods instead of the Button() method.

id = Hand Terminal ID (1-65535).

pos = Position of inputfield. ( 0-239 )

txt = Text for a button.

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL ClearCmd( long id, short cmd, short start, short stop); ( METHOD )

Partially clears the display of any text or field

id = Hand Terminal ID (1-65535).

cmd

- Bit 0 	CLEAR_TXT Removes text from <start> to <stop>

- Bit 1	CLEAR_FLD Removes fields from <start> to <stop>

- Bit 2	CLEAR_FLDDATA Clears the field data from <start> to <stop>

- Bit 3-7	Reserved

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL FieldCmd(long id, short pos, short fcmd); ( METHOD )

Definition of the field function.

id = Hand Terminal ID (1-65535).

pos = Position of inputfield. ( 0-239 )

fcmd

- Bit 0	FLD_REMOVE Removes the field

- Bit 1	FLD_CLEAR Clears the field (from locked fields also)

- Bit 2	Reserved

- Bit 3	FLD_LOCK Locks the field (cannot be written to)

- Bit 4	Reserved

- Bit 5	Reserved

- Bit 6	Reserved

- Bit 7	FLD_ACTIVE Field is set to active.

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL FldTxt(short id, short pos, LPCTSTR txt); ( METHOD )

Fills (presets) the field with the text. If the length of txt is more than the length of the field, the excess txt will be corrupted. If the field is already filled with text, it will be replaced with new text.

id = Hand Terminal ID (1-65535).

pos = Position of inputfield. ( 0-239 )

txt = Text for the field.

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL NewField(short id, short pos, short field_len, short style); ( METHOD )

id = Hand Terminal ID (1-65535).

pos = Position of inputfield. ( 0-239 )

field_len = field length

style

- Bit 0	SND_ENTER Field is sent to HOST by pressing the OK key.

- Bit 1	NO_SEND Field is not sent if the SND_ALL command occurs. Field will be sent, if the field itself gave the SND_ALL command.

- Bit 2	SND_ALL All fields in the page, (except NO_SEND fields) are sent to the host when the OK key is pressed.

- Bit 3	FLD_LOCK Field is locked. Field cannot be written to (used for "button" style fields. )

- Bit 4	FLD_LINE Field is underlined. _ _ _ _ _ _ _ _ _

- Bit 5	FLD_READER Field can be filled with laser- or external scanner data.

- Bit 6	FLD_CLR Field is cleared if ACK is received.

- Bit 7	FLD_ACTIVE Field is active ( it has a cursor ).

- SND_ENTER and FLD_READER combinations:

| - SND_ENTER | - FLD_READER | - FUNCTION |
| --- | --- | --- |
| - 0 | - 0 | - Field is not sent by pressing the OK key, and cannot be read with a laser scanner. |
| - 0 | - 1 | - Field is not sent by pressing the OK key, but it can be filled from the keyboard and with a laser scanner. |
| - 1 | - 0 | - Field is sent by pressing the OK key, but cannot be filled with a laser scanner. |
| - 1 | - 1 | - Field is sent by pressing the OK key or by reading with a laser scanner. |

- A maximum of 15  input fields can be defined for one form.

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL PopMessage( long id, short pos, LPCTSTR txt); ( METHOD )

This method defines a message which is displayed as long as any key is pressed. Only one PopMessage method call can be included in the sendbuffer

id = Hand Terminal ID (1-65535).

pos = Start position of message. ( 0-79 )

txt = Text for the field.( may include VT and CR characters. )

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL SetView(long id, short row); ( METHOD )

Sets the Hand terminal display view. Generally the display view will be set automatically when fields are edited.

id = Hand Terminal ID (1-65535).

row = View line number ( 0-8 ). If this value is greater than 8, this method will be corrupted.

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL Text(long id, short pos, LPCTSTR txt);

This methods defines a text string to specific position.

id = Hand Terminal ID (1-65535).

pos = Start position of text. ( 0-239 )

txt = Text for the field.

Return value TRUE if succesful. FALSE if sendbuffer is full.

BOOL What(long id, short delay); ( METHOD )

Hand terminal send WHAT command (event) to HOST when delay has expired.

id = Hand Terminal ID (1-65535).

delay = 1 - 99 seconds

Return value TRUE if succesful. FALSE if sendbuffer is full.

About Dialog appears at the header by mouse click.

Driver control shows handterminal ID to the box.

Frame Info Dialog shows transaction details.

Data from hand terminal <DataFromHsField>dterminal

Commands in sendbuffer

Frame Info Dialog appears at the box by mouse click.